# KiZAN IoT Workshop

Lab Exercises

# Lab 4 – Temperature Sensor

## Goal

The goal of this exercise is to build a simple circuit to read from a temperature sensor using a Windows IoT Core device (Raspberry Pi 2, in our case) and an app built and debugged with Visual Studio 2015.

## Requirements

Prior to starting this exercise, please ensure that you meet the following requirements.

### Hardware

Please verify that you have the following components:

- Raspberry Pi 2 Model B w/ EDIMAX Wifi Adapter

- Wall Adapter Power Supply

- Raspberry Pi GPIO Ribbon Cable

- SparkFun Pi Wedge Breakout Board

- Breadboard

- (1) TMP36 Temperature Sensor

- (1) MCP3008 Analog to Digital Converter

- (1) 0.1uF ceramic capacitor

- Jumper Wires M/M (as required)

### Software

Please verify that you have the following software installed on your Windows 10 laptop:

- Visual Studio 2015 Community Edition (or greater)

https://www.visualstudio.com/vs-2015-product-editions

- Windows 10 IoT Core Dashboard

https://developer.microsoft.com/en-us/windows/iot/downloads

### Networking

To communicate with the Raspberry Pi 2, you must be connected to the following Wifi network:

| SSID | Password |
| --- | --- |
| iotlab | p@ssw0rd |

## Part 1 – Circuit Assembly

- Carefully attach the SparkFun Pi Wedge Breakout Board to the Breadboard, ensuring that the pins on the Pi Wedge align with the pin holes on the Breadboard.

Figure 1 - SparkFun Pi Wedge connected to the Breadboard

- Using (2) Black Jumper Wires, connect the 3.3V and GND pins on the SparkFun Pi Wedge to the positive (*) and negative (-) rails on the side of the breadboard, respectively.

Figure 2 - 3.3V and GND connected to the Breadboard

- Carefully place the MCP3008 Analog to Digital Converter on the breadboard.  The dimple on the top of the chip should be located on the bottom-left after it is placed on the breadboard.

Figure 3 - MCP3008 Analog to Digital Converter

- Using (2) Red Jumper Wires, connect both pin 16 (VDD) and pin 15 (VREF) of the MCP3008 to the positive (+) rail on the breadboard.

Figure 4 – MCP3008 pins 15 and 16 connected to 3.3V

- Using (2) Black Jumper Wires, connect both pin 14 (AGND) and pin 9 (DGND) of the MCP3008 to the negative (-) rail on the breadboard.

Figure 5 - MCP3008 pins 9 and 14 connected to GND.

- Using a Yellow Jumper Wire, connect pin 13 (CLK) to the SCK pin of the SparkFun Pi Wedge.

Figure 6 - MCP3008 pin 13 (CLK) connected to SparkFun Pi Wedge pin SCK.

- Using a White Jumper Wire, connect pin 12 (DOUT) to the MISO pin of the SparkFun Pi Wedge.

Figure 7 – MCP3008 pin 12 (DOUT) connected to SparkFun Pi Wedge pin MISO.

- Using a Blue Jumper Wire, connect pin 11 (DIN) to the MOSI pin of the SparkFun Pi Wedge.

Figure 8 - MCP3008 pin 11 (DIN) connected to SparkFun Pi Wedge pin MOSI.

- Using a Green Jumper Wire, connect pin 10 (CS) to the CE0 pin of the SparkFun Pi Wedge.

Figure 9 - MCP3008 pin 10 (CS) connected to SparkFun Pi Wedge pin CE0.

- Connect the TMP36 Temperature Sensor to the breadboard as shown.  Be sure to orient the temperature sensor appropriately.

Figure 10 - TMP36 Temperature Sensor connected to the breadboard.

- Connect the 0.1uF ceramic capacitor across the left two pins of the temperature sensor (GND and VOUT, respectively).

Figure 11 - 0.1 uF Capacitor connected to the breadboard.

- Using a Black Jumper Wire, connect the left-most pin (GND) of the TMP36 Temperature Sensor to the negative (-) rail of the breadboard.

Figure 12 - TMP36 GND pin connected to GND.

- Using a Red Jumper Wire, connect the right-most pin (+VS) of the TMP36 Temperature Sensor to the positive (+) rail of the breadboard.

Figure 13 - TMP36 +VS pin connected to 3.3V.

- Using a Yellow Jumper Wire, connect the middle pin (VOUT) of the TMP36 Temperature Sensor to pin 1 (CH0) of the MCP3008 Analog to Digital Converter.

Figure 14 - TMP36 VOUT connected to MCP3008 pin 1 (CH0).

## Part 2 – Code

- Connect the Wall Adapter Power Supply to the micro USB port on the Raspberry Pi.  Next, plug in the Wall Adapter Power Supply.

- Open the Windows 10 IoT Core Dashboard application.  Once your device has completed its power up sequence, you should see your device in the list of My devices.

Figure 15 - Windows 10 IoT Core Dashboard

- Using Visual Studio 2015, open the Lab04.sln solution from the downloaded source package.  Lab04.sln is located in the KiZAN-IoT-Workshop\src\Lab04 folder.

- Verify that the Debug configuration and ARM platform are selected.

Figure 16 - Build Configuration and Platform

- Next, change Run from Device to Remote Machine.

Figure 17 - Change Run from Device to Remote Machine

- In the Remote Connections dialog box, find your device in the list of Auto Detected devices.  Select your device, and click the Select button.

Figure 18 - Remote Connections

- Press F5 to run the application.  Visual Studio will restore any missing NuGet packages, build the application, and then deploy the application to the remote machine.

- With the application running, open the Output window in Visual Studio (View \ Output).

- Verify that the current temperature is written to the Output window.

Figure 19 - Output Window

- Stop debugging.

- Using Windows 10 IoT Core Dashboard, click the globe icon in the Open in Device Portal column for your device.

- When prompted enter the following User Name and Password.

| User Name | Password |
| --- | --- |
| Administrator | p@ssw0rd |

- In the upper, right-hand corner, click Power, and then click Shut down.

Figure 20 - Power Menu

- When prompted to confirm, press OK.

- It is safe to remove power when the Device Portal prompts to “Restart the device to reconnect”.